Things to consider:

Budget:

- Cost of engine

- Cost of additional hardware and drilling tools

Structure:

- How the engine will physically connect

- Minimum 5:1 thrust weight ratio

    - Weight of materials used

    - Weight of electronics

    - Weight of the actual engine

RDDT/UDDT:

Rocket delay drilling tool

Universal delay drilling tool

Used to time chute deployment of engine by cutting off segments of the engine starter

DMS/RMS:

Disposable Motor System (Single use)

Reusable Motor System (Multi use)

|  | Super Thunder | Blue Thunder | Redline |
| --- | --- | --- | --- |
| Cost | $79 | $93 | $101 |
| Motor System | DMS | RMS | RMS |
| Motor Diameter | 38mm | 38mm | 38mm |
| Casing Length | 203mm | 213mm | 337mm |
| Additional Hardware Requirements | N/A | RMS-38/360 | RMS-38/720 |
| Total Impulse | 320N-sec | 600N-sec | 597.3N-sec |
| Average thrust | 550N | 461N | 612N |
| Peak Thrust | 667N-sec | 734N-sec | 811.7N-sec |
| Thrust Duration | 0.57sec | 1.3sec | 1.1sec |
| Delay Time | up to 14 sec | up to 14 sec | up to 14 sec |
| Delay Drilling tool | UDDT | 38mm RDDT | 38mm RDDT |
| Propellant Weight | 145 g | 277 g | 324g |
| Motor Weight | 315 g | 514 g | 602g |

Super Thunder:

https://aerotech-rocketry.com/products/product_a076ec4b-9994-a841-e9f9-889bac3bddc3

Blue Thunder:

https://aerotech-rocketry.com/products/product_aca01942-6e02-d75c-25c2-6cf53a6065e5?_pos=1&_sid=3c919e679&_ss=r

Redline:

https://www.csrocketry.com/rocket-motors/aerotech-rocketry/motors/38mm/38/720-reloads/aerotech-i600r-14a-redline-rocket-motor.html